- IP Camera Supporting Matrix 攝影機支援矩陣規格

| 廠牌Vendor | Connectivity | Video | Audio | Additional Features | 型號 |
| --- | --- | --- | --- | --- | --- |
| AXIS | RTSP | RTSP | RTSP | Cv, Ms, Mv, PT8, PTf, Z, F, L, Di, Do | 209FD, 214, 211M, 241Q, Q7401 |
| IQinVision | HTTP | M-JPEG | --- | Mv | IQ755, IQ753 |
| CTC-U | UDP, RTSP | RTSP | RTSP |  | DVS-8504E |
| ACTi | SDK-10000 | MPEG-4 | PCM | Cv, Ms, Mv, PT8, Z, L, Di, Do | ACD-2200, ACM-1310N |
| VIVOTEK | VNDP-080610 | MPEG-4, VV_H263, M-JPEG | AAC, AMR | Ms, Mv, PT4, Z | IP7135, PT3117, PZ6112, PZ7151, FD7131, VS7100, PZ6114, PT7135, IP7142, FD6111, FD7131 |
| ZAVIO | RTSP, HTTP | RTSP, MPEG-4, M-JPEG, | RTSP, G.711 | Cv | F312A, F721A, F210A |
| SONY | HTTP | MPEG-4, M-JPEG, H.264 | G.711, G.726 | Ms, Mv[*1], PT8, Z | SNC-RZ50N, RZ30, P5, SNC-RX550 |
| Arecont Vision | AV2000 2.5.37 | M-JPEG | --- | Cv | AV3130, AV5100 |
| EtroVision | EtroSDK 1.31 | MPEG-4 | --- | Cv, PT4, Z, F, L | VTC-460 |
| JVC V.Networks | HTTP | M-JPEG | --- | PT8 | C30U, C625U, C655U |
| BOSCH VideoJet | HTTP | M-JPEG | --- | PT8 | VideoJet-10 |

- RTSP Supporting Information

| Supported Video Codecs | MPEG-4, M-JPEG, H.264 |
| --- | --- |
| Supported Audio Codecs | G.711, AAC, AMR, G.726 |
| Supported Streaming Modes | UDP, TCP, HTTP |

- Additional Features Description

| Feature | Description |
| --- | --- |
| PT4 | Pan-Tilt with 4 directions |
| PT8 | Pan-Tilt with 8 directions |
| PTf | Pan-Tilt with without direction restriction (free mode) |
| Z | Zoom (by wheel) |
| F | Focus control near/far |
| L | Preset location get/set |
| Cv | Configure camera video parameter (resolution, fps, etc) |
| Mv | Motion detection event |
| Ms | Motion detection settings |
| Di | Digital input (DI) trigger |
| Do | Digital output (DO) control |
| A | Auto-pan |

- Remarks

- VV_H263: A variation of proprietary H263 developed apparently by VIVOTEK.

- [*1] Not all models are supported. Known supported models are RZ50, RX550

- AVI-Export Supporting Information

| Supported Video Codecs | MPEG-4, M-JPEG, H.264 |
| --- | --- |
| Supported Audio Codecs | G.711, PCM |